| HBF Health and Safety Charter Protecting our Customers, Visitors and Neighbours (supplementary notes for guidance) |  |  |
| --- | --- | --- |
| 1 Standard Guidelines for Public Protection |  |  |
| Heading/Element | Principal Considerations | Further Considerations |
| Education |  |  |
| Letters | Write to the following people/groups once the development programme has been established and the method of working is known: - Neighbours affected - Local Authorities - Police - Other Emergency Services if likely to be affected - Schools The letter should include, names and telephone numbers of Company contacts both on site and at office, scope of works, agreed working hours, any offsite works and traffic management Write again if circumstances necessitate any abnormal loads, lifting operations or restrictions to pedestrian or vehicle movements Should any changes occur, write to all again explaining change in circumstance and reason Sub-contractors and suppliers are to be advised of any vehicle restrictions such as, permitted delivery times and working hours Include details in pre-construction Health and Safety Plan, pre-contract order and pre-start meeting | Consider the following groups:- - Local Councillors - Residents Groups - Local Shopkeepers Offer to do presentations to schools or clubs etc, outlining the development and the hazards of building sites Operate under a "Considerate Constructor" or equivalent scheme |
| # Induction |  | Consider getting visitors to sign a form to record they have been inducted Should visitors details be recorded as they enter or leave the site? |
| Physical exclusion | Where particular hazards exist, these areas are to be fenced off, and warning signs strategically placed so as to exclude unauthorised access Site perimeter fencing is to be a minimum of 2 metres high close mesh fencing, or better, mechanically secured where construction activity is being undertaken. Company personnel are to be authorised to take a robust view against unauthorised access and take immediate steps to escort any intruders from the site, without endangering themselves, calling for back-up if necessary The working area should be secured against unauthorised access where practical | Unaccompanied visits should be prohibited. Escorting people from site, especially if they are customers or potential customers can be difficult. It must be executed firmly but politely and always with an explanation. Arrange a suitable appointment when an accompanied visit can be made Regular out of hours inspections by management. |
| Lighting | Suitable temporary lighting may be installed and maintained | Before occupation of any dwelling seek to provide the permanent lighting to that dwelling from the public highway. |
| Tidiness | Sites are to be kept tidy to promote safe access and movement |  |

Document revision 20th April 2006