| HBF Health and Safety Charter Protecting our Customers, Visitors and Neighbours (supplementary notes for guidance) |  |  |
| --- | --- | --- |
| 3 Develop Standard Guidelines for Traffic Management and Pedestrian Safety |  |  |
| Heading/Element | Principal Considerations | Further Considerations |
| Traffic Management - off-site | These will necessarily be worked up in conjunction with the Highway Authority and if necessary the Police Things to consider are: - Special circumstances such as nearby fire stations, hospitals, schools, stadiums, pedestrian crossings, traffic lights etc - Existing speed limits on existing roads - Width of existing carriageways - Existing parking arrangements If it is decided that any additional restrictions will be necessary as a result of the development, statutory processes will apply and must be adhered to | Advise all those detailed in the "Letters" section above |
| Traffic Management - on-site | A detailed Traffic Management Plan must be drawn up and displayed on site. Its operation must form part of the Induction for persons working on, making deliveries to and collections from the site The Traffic Management Plan must be kept "live" and up to date and must be adapted to suit changing circumstances, especially just prior to purchaser occupation The Traffic Management Plan must include safe segregation of pedestrians from site vehicles The Traffic Management Plan must show vehicle turning areas and areas for unloading and storage of material The traffic Management Plan must either show where visitors/customers may park safely on site, or state that no parking is available Reversing areas are to be under supervised control. Suitable direction signage is to be displayed in prominent positions. | The Traffic Management Plan could be displayed on the site boundary fencing Consideration is to be given to traffic calming and speed bumps Consideration is to be given to one-way systems. These have to be practical to be workable and should be avoided if short-cuts are inevitable Consideration to be given to gated access or barriers at points of access/egress. If there is no on-site parking for visitors and/or customers, a suggested location for parking elsewhere could be stated |
| Speed limits | Determine suitable and appropriate speed limits for the site. Once determined these are to be displayed prominently. Speed limits may need to change from time to time so should be reviewed periodically | A guideline of 10mph reducing to 5mph at high risk points Consideration may be given to governing speeds of plant in special circumstances Consideration to be given to marshalling vehicles by trained operatives where risk assessment confirms the need. |
| Craneage | Consideration to be given to over-sailing neighbours, public areas and occupied units when establishing lifting plans for tower cranes and mobile cranes |  |
| Party walls | Consideration to be given to neighbouring properties and Party Walls when preparing the Health and Safety Plan |  |
| Scaffolding | Where the development abuts an existing Public Highway consideration is to be given to scaffolding over walkways etc with all the necessary permissions involved |  |

Document revision 20 April 2006